# CAB-1 Remote AUX1 Keys

# Applying Power to Sidings

- Turn off siding transformer: TR-5-AUX1-0

- Toggle the routing of transformer power to your siding:  ACC-<siding#>-AUX1

NOTE:  This toggles the routing of power.  If power was already routed to your siding, then executing this will un-route it.  So you might need to do it twice if it was already routed to begin with…

- Apply power to your siding:

TR-5-BOOST (command) –or-

TR-5-<dial up power> (conventional)

NOTE:  If you don’t get power on your siding, execute step 2 again…

# Siding Operation
Conventional Mode

- Turn off power to sidings: TR-5-AUX1-0.

- Change switches to route trains onto your siding.

- Set up new train on siding.

- Route power to your siding: ACC-siding#- AUX1

- Stop mainline train at entrance to siding and put into neutral after reverse: TR-main#-DIR-DIR-DIR.  Leave power on your mainline.

- Start siding train, TR-5-BOOST, dial in some power and move train out slowly.  The train will keep moving on the main line.

- Once on the mainline, address track: TR-main#, and control speed as desired.  Slow down track at end of train.

- Once clear of siding, hit DIR, Mainline train moves onto siding and press TR-5 to control in the siding.

- Turn off power to siding: TR-5-AUX1-0

- Change switches on mainline to run new train.

# Switching Transformers 
Conventional <-> Command

Mainline Tracks 1 and 4 & Siding power just turn the switch on Lionel PowerMaster for that track.

Mainline Track 2 -> Conventional: TR-2-M-SET

Mainline Track 2 -> Command: TR-2-L-SET

Mainline Track 3 -> Conventional: TR-3-M-SET

Mainline Track 3 -> Command: TR-3-L-SET

Tracks 2 & 3 Command Mode: one of the lights on the IC controller in the transformer box will flash slowly.

All Transformers:  TR-<track#>-AUX1-0 kills power

# TMCC: Setting Engine ID

- Kill power on your track:  TR-<track#>-AUX1-0

- Slide the engine switch from “Run” to “Prog”.  This switch is usually located underneath the engine somewhere.

- Place engine on track.

- Power up track:  TR-<track#>-AUX1-BOOST.  Your engines lights will be on at this point.

- Set the engine ID:  ENG-<eng#>-SET.  Your engine will sound its horn/whistle and flash its lights if it received the command.

- Kill power on your track:  TR-<track#>-AUX1-0

- Slide the engine switch from “Prog” back to “Run”.

- Place engine back on track.

- Power up track: TR-<track#>-AUX1-BOOST.

- Run some trains!! TR-<eng#> and dial up speed/use whistle/etc…

# TMCC: Creating Lash-ups

- Kill power on your track:  TR-<track#>-AUX1-0

- Pick a lash-up number that is not in use. TR 6-9 are available for lash-ups (1-4 are for mainlines, 5 is for sidings).

- To set the lead engine: 
TR-<lash#>-<eng#>-F-SET

- To set a middle engine that is forward-facing:
TR-<lash#>-<eng#>-SET

- To set a middle engine that is backward-facing:
TR-<lash#>-<eng#>-DIR-SET

- To set the tail engine forward-facing:
TR-<lash#>-<eng#>-R-SET

- To set the tail engine backward-facing:
TR-<lash#>-<eng#>-R-DIR-SET

Now you can address TR-<lash#> to control the entire lash-up.

To Remove an engine from a lash-up: 
TR-0-<eng#>-SET

NOTE: You should always remove all engines in a lash-up when you’re done operating. Trust me on this!